# Worksheet 4

Student Name: Anjali kumari	UID:25MCA20023

Branch: MCA(General)	Section/Group:25MCA-1-A

Semester: III	Date of Performance:09/09/26

Subject Name: Internet Of Things	Subject Code: 25CAH-708

## Aim/Overview of the practical:

Interface an Arduino Uno, Led and Buzzer with RFID reader/writer MFRC522 and register the card's UID. Write a program to check whether the card used is authorized or unauthorized.

  - If the card is authorized, then Turn ON only "Yellow Led" and buzzer will Beep twice.

  - If the card is unauthorized then Turn ON only “Blue Led" and buzzer will Beep three times.

## Apparatus:

Hardware Requirements: Breadboard

MFRC 522 RFID Sensor

Buzzer Jumper wires

LED (2 pieces) Arduino Uno Board

Software requirements: Arduino IDE

Laptop

## Circuit Diagram(TinkerCad):

Coding:

#include <SPI.h> #include <MFRC522.h>

// RFID pins #define SS_PIN 10

#define RST_PIN 9

// Output pins

#define YELLOW_LED 6

#define BLUE_LED 7

#define BUZZER 8

MFRC522 rfid(SS_PIN, RST_PIN);

byte authorizedUID[] = {0xA3, 0xB4, 0xC5, 0xD6};

void setup() { Serial.begin(9600); SPI.begin(); rfid.PCD_Init();

pinMode(YELLOW_LED, OUTPUT); pinMode(BLUE_LED, OUTPUT); pinMode(BUZZER, OUTPUT);

digitalWrite(YELLOW_LED, LOW); digitalWrite(BLUE_LED, LOW); digitalWrite(BUZZER, LOW);

Serial.println("RFID Reader Ready..."); Serial.println("Place your RFID card near the reader.");

}

void loop() {

// Check whether a new card is present if (!rfid.PICC_IsNewCardPresent()) { return;

}

// Read the card

if (!rfid.PICC_ReadCardSerial()) { return;

}

// Print UID Serial.print("Card UID: ");

for (byte i = 0; i < rfid.uid.size; i++) { if (rfid.uid.uidByte[i] < 0x10) { Serial.print("0");

}

Serial.print(rfid.uid.uidByte[i], HEX); Serial.print(" ");

}

Serial.println();

// Check whether UID is authorized if (checkUID()) {

Serial.println("AUTHORIZED CARD");

// Yellow LED ON digitalWrite(YELLOW_LED, HIGH); digitalWrite(BLUE_LED, LOW);

// Beep twice

for (int i = 0; i < 2; i++) { digitalWrite(BUZZER, HIGH); delay(200);

digitalWrite(BUZZER, LOW); delay(200);

}

delay(2000);

// Turn LED OFF digitalWrite(YELLOW_LED, LOW);

} else {

Serial.println("UNAUTHORIZED CARD");

// Blue LED ON digitalWrite(YELLOW_LED, LOW); digitalWrite(BLUE_LED, HIGH);

// Beep three times

for (int i = 0; i < 3; i++) { digitalWrite(BUZZER, HIGH); delay(200); digitalWrite(BUZZER, LOW); delay(200);

}

delay(2000);

// Turn LED OFF digitalWrite(BLUE_LED, LOW);

}

// Stop communication with current card rfid.PICC_HaltA(); rfid.PCD_StopCrypto1();

delay(500);

}

bool checkUID() {

// First check UID length

if (rfid.uid.size != sizeof(authorizedUID)) { return false;

}

// Compare every byte

for (byte i = 0; i < rfid.uid.size; i++) {

if (rfid.uid.uidByte[i] != authorizedUID[i]) { return false;

}

}

return true;

}

6.Output:

FIG1: Interfacing of RFID WITH  BUZZOR AND LED ON ARDINOUNO

## Learning outcomes (What I have learnt):

  - RFID sensor and its components.

  - Interfacing of Arduino uno board with RFID sensor.

  - Differences between RFID card and reader, in the context of active and passive elements.