# Worksheet 2

Student Name: Anindita Dhar	UID: 25MCA20259

Branch: MCA(General)	Section/Group:25MCA-1-A

Semester: III	Date of Performance:7/8/26

Subject Name: Internet Of Things	Subject Code: 25CAH-708

## Aim/Overview of the practical:

Interface a led, pushbutton (Pull Up) and LCD with Arduino Uno. Use two led and two pushbuttons. Use pushbutton as toggle switch.

  - Case 1: When press button 1 once then LED 1 will turn ON and print “Button1 ON” in Row 1 and “LED 1 ON” in Row 2.

  - Case 2: When we press button 1 again then LED 1 will turn OFF and print “Button1 OFF” in Row 1 and “LED 1 OFF” in Row 2.

  - Case 3: When we press button 2 once then LED 2 will turn ON and print “Button2 ON” in Row 1 and “LED 2 ON” in Row 2.

  - Case 4: When we release button 2 again then LED 2 will turn OFF and print “Button2 OFF” in Row 1 and “LED2 OFF” in Row 2.

## Apparatus

Hardware Requirements: Arduino Uno R3 Board

LCD LED

Pushbutton

Software requirements: Microsoft Windows 10

TinkerCAD

- Circuit Diagram(TinkerCad):

Case 1:

## Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

const int button1 = 7; const int button2 = 8; const int led1 = 9; const int led2 = 10; bool led1State = false;

int lastButton1 = HIGH; int lastButton2 = HIGH;

void setup()

{

pinMode(button1, INPUT_PULLUP); pinMode(button2, INPUT_PULLUP);

pinMode(led1, OUTPUT); pinMode(led2, OUTPUT);

lcd.begin(16, 2);

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Ready");

digitalWrite(led1, LOW); digitalWrite(led2, LOW);

}

void loop()

{

int b1 = digitalRead(button1); int b2 = digitalRead(button2);

// BUTTON 1 PRESSED

if (lastButton1 == HIGH && b1 == LOW)

{

led1State = !led1State; digitalWrite(led1, led1State); lcd.clear();

if (led1State)

{

lcd.setCursor(0, 0); lcd.print("Button1 ON");

lcd.setCursor(0, 1);

lcd.print("LED 1 ON");

}

delay(200);

}

lastButton1 = b1;

}

## Output:

Case 2:

## Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

const int button1 = 7; const int button2 = 8; const int led1 = 9; const int led2 = 10;

bool led1State = false;

int lastButton1 = HIGH; int lastButton2 = HIGH;

void setup()

{

pinMode(button1, INPUT_PULLUP); pinMode(button2, INPUT_PULLUP); pinMode(led1, OUTPUT); pinMode(led2, OUTPUT);

lcd.begin(16, 2);

lcd.clear(); lcd.setCursor(0, 0); lcd.print("Ready");

digitalWrite(led1, LOW); digitalWrite(led2, LOW);

}

void loop()

{

int b1 = digitalRead(button1); int b2 = digitalRead(button2);

// BUTTON 1 PRESSED AGAIN

if (lastButton1 == HIGH && b1 == LOW)

{

led1State = false; digitalWrite(led1, LOW); lcd.clear(); lcd.setCursor(0, 0); lcd.print("Button1 OFF"); lcd.setCursor(0, 1);

lcd.print("LED 1 OFF");

delay(200);

}

lastButton1 = b1;

}

## Output:

### Case 3:

Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

const int button1 = 7; const int button2 = 8; const int led1 = 9; const int led2 = 10;

bool led1State = false;

int lastButton1 = HIGH; int lastButton2 = HIGH; void setup()

{

pinMode(button1, INPUT_PULLUP); pinMode (button2, INPUT_PULLUP); pinMode (led1, OUTPUT);

pinMode (led2, OUTPUT);

lcd.begin(16, 2);

lcd.clear(); lcd.setCursor(0, 0); lcd.print("Ready"); digitalWrite(led1, LOW); digitalWrite(led2, LOW);

}

void loop()

{

int b1 = digitalRead(button1); int b2 = digitalRead(button2);

// BUTTON 2 PRESSED

if (lastButton2 == HIGH && b2 == LOW)

{

digitalWrite(led2, HIGH); lcd.clear(); lcd.setCursor(0, 0); lcd.print("Button2 ON"); lcd.setCursor(0, 1);

lcd.print("LED 2 ON");

}

lastButton1 = b1; lastButton2 = b2; delay(50);

}

### Output:

### Case 4:

Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2); const int button1 = 7;

const int button2 = 8; const int led1 = 9; const int led2 = 10;

bool led1State = false;

int lastButton1 = HIGH; int lastButton2 = HIGH; void setup()

{

pinMode(button1, INPUT_PULLUP); pinMode(button2, INPUT_PULLUP); pinMode(led1, OUTPUT); pinMode(led2, OUTPUT);

lcd.begin(16, 2);

lcd.clear(); lcd.setCursor(0, 0); lcd.print("Ready");

digitalWrite(led1, LOW); digitalWrite(led2, LOW);

}

void loop()

{

int b1 = digitalRead(button1); int b2 = digitalRead(button2);

// BUTTON 2 RELEASED

if (lastButton2 == LOW && b2 == HIGH)

{

digitalWrite(led2, LOW); lcd.clear(); lcd.setCursor(0, 0); lcd.print("Button2 OFF"); lcd.setCursor(0, 1); lcd.print("LED2 OFF");

}2

lastButton1 = b1; lastButton2 = b2; delay(50);

}

### Output:

## Learning outcomes (What I have learnt):

  - Using a 16*2 LCD in circuit.

  - Circuit formation using Arduino uno R3, push button, LED, LCD and potentiometer.

  - Integrating a potentiometer in a circuit.

  - Concepts of pull up and pull down in push button.

  - Components of 16*2 LCD,