# Worksheet 3

Student Name: Anindita Dhar	UID:25MCA20259

Branch: MCA(General)	Section/Group:25MCA-1-A

Semester: III	Date of Performance: 22/08/2026

Subject Name: Internet Of Things	Subject Code: 25CAH-708

#### Aim/Overview of the practical:

Interface an Arduino Uno with Ultrasonic Sensor to calculate the distance and print on a Serial monitor and on LCD. Follow the mentioned cases after the calculation of distance.

  - If the distance is > 250 cm green light is in ON condition.

  - If the distance is < 250 cm and the distance > 90 yellow light is in ON condition and the green light remains Off

  - If the distance is < 90 cm turn on the buzzer and lights will remain OFF.

#### Apparatus:

Hardware Requirements: Arduino Uno Board

Ultrasonic Sensor LCD (16*2)

Software requirements:	TinkerCAD

MS Windows 10

#### Circuit Diagram(TinkerCad):

(Case 1) Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

// Ultrasonic sensor const int trigPin = 9; const int echoPin = 10;

// Green LED

const int greenLED = 6; void setup(){

pinMode(trigPin, OUTPUT); pinMode(echoPin, INPUT); pinMode(greenLED, OUTPUT);

Serial.begin(9600); lcd.begin(16, 2);

lcd.print("Distance:");

}

void loop()

{

digitalWrite(trigPin, LOW); delayMicroseconds(2); digitalWrite(trigPin, HIGH); delayMicroseconds(10); digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH); float distance = duration * 0.0343 / 2; Serial.print("Distance: "); Serial.print(distance);

Serial.println(" cm");

lcd.setCursor(0, 0); lcd.print("Distance:	"); lcd.setCursor(0, 1); lcd.print(distance); lcd.print(" cm	");

if (distance > 250){ digitalWrite(greenLED, HIGH); Serial.println("GREEN LED ON");

}

else{

digitalWrite(greenLED, LOW);

}

delay(500);

}

### OUTPUT:

#### (Case 2) Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

// Ultrasonic sensor const int trigPin = 9; const int echoPin = 10;

// Yellow LED

const int yellowLED = 7; void setup()

{

pinMode(trigPin, OUTPUT); pinMode(echoPin, INPUT); pinMode(yellowLED, OUTPUT); Serial.begin(9600);

lcd.begin(16, 2);

lcd.print("Distance:");

}

void loop()

{

// Send ultrasonic pulse digitalWrite(trigPin, LOW); delayMicroseconds(2); digitalWrite(trigPin, HIGH); delayMicroseconds(10); digitalWrite(trigPin, LOW);

// Measure echo time

long duration = pulseIn(echoPin, HIGH);

// Calculate distance in cm

float distance = duration * 0.0343 / 2;

// Print on Serial Monitor Serial.print("Distance: "); Serial.print(distance); Serial.println(" cm");

// Print on LCD lcd.setCursor(0, 0); lcd.print("Distance:	"); lcd.setCursor(0, 1); lcd.print(distance); lcd.print(" cm	");

if (distance < 250 && distance > 90)

{

digitalWrite(yellowLED, HIGH); Serial.println("YELLOW LED ON");

}

else

{

digitalWrite(yellowLED, LOW);

}

delay(500);

}

### OUTPUT:

#### (Case 3) Coding:

#include <LiquidCrystal.h> LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

// Ultrasonic sensor const int trigPin = 9; const int echoPin = 10;

// Buzzer

const int buzzer = 8;

void setup()

{

pinMode(trigPin, OUTPUT); pinMode(echoPin, INPUT); pinMode(buzzer, OUTPUT); Serial.begin(9600); lcd.begin(16, 2);

lcd.print("Distance:");

}

void loop()

{

digitalWrite(trigPin, LOW); delayMicroseconds(2); digitalWrite(trigPin, HIGH); delayMicroseconds(10); digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH); float distance = duration * 0.0343 / 2; Serial.print("Distance: "); Serial.print(distance);

Serial.println(" cm");

lcd.setCursor(0, 0); lcd.print("Distance:	"); lcd.setCursor(0, 1); lcd.print(distance); lcd.print(" cm	");

if (distance < 90)

{

digitalWrite(buzzer, HIGH); Serial.println("BUZZER ON");

}

else

{

digitalWrite(buzzer, LOW);

}

delay(500);

}

### OUTPUT:

## Learning outcomes (What I have learnt):

- Ultrasonic sensors and its properties.

- Interfacing of ultrasonic sensors with Arduino Uno and LCD.

- Connecting ultrasonic sensor ,Arduino Uno board and LCD in a circuit.