Student Name: Ashi Gupta Branch: MCA (General) Semester: 3rd

Subject Name: IOT Lab

# Worksheet 5

UID: 25MCA20160

Section/Group: 25MCA-1A

Date of Performance: 18/09/2026 Subject Code: 25CAH-708

-------------------------------------------------------------------------------------------------------------------

## Aim/Overview of the practical: (EVEN UIDS)

Interface an Arduino Uno with Temp Sensor to calculate the temperature in Fahrenheit and print on a Serial monitor and on LCD. Follow the mentioned cases after the calculation of distance.

  - If the temp is > 70-degree F green light is in ON condition.

  - If the temp is<70-degree F and temp is> 30-degree F yellow light is in ON condition and the green light remains Off

  - If the temp is < 30 degree F  turn on the buzzer and the lights will remain OFF.

## Apparatus:

- Hardware Requirements:

  - Arduino Uno R3 – 1,

  - Breadboard -1,

  - LED – 2 (Yellow, green),

  - Resistor (220Ω) 2,

  - Buzzer – 1,

  - TMP36,I2c

  - Jumper Wires,

  - USB Cable/Power Supply

## Software requirements:

  - Arduino IDE,

  - CH340,

## Code & Circuit Diagram(TinkerCad):

#include <Wire.h>

#include <LiquidCrystal_I2C.h> LiquidCrystal_I2C lcd(0x27, 16, 2); int tempPin = A0;

int greenLED = 4; int yellowLED = 6; int buzzer = 8; void setup() {

Serial.begin(9600); lcd.init(); lcd.backlight();

pinMode(greenLED, OUTPUT);

pinMode(yellowLED, OUTPUT); pinMode(buzzer, OUTPUT); lcd.setCursor(0, 0); lcd.print("Temperature:");

}

void loop() {

// Read TMP36

int value = analogRead(tempPin);

// Convert analog value to voltage float voltage = value * 5.0 / 1023.0;

// TMP36 -> Celsius

float temperatureC = (voltage - 0.5) * 100.0;

// Celsius -> Fahrenheit

float temperatureF = (temperatureC * 9.0 / 5.0) + 32.0;

// Serial Monitor Serial.print("Temperature: "); Serial.print(temperatureF); Serial.println(" F"); lcd.setCursor(0, 1); lcd.print("Temp: "); lcd.print(temperatureF, 1); lcd.print(" F ");

// First turn everything OFF digitalWrite(greenLED, LOW); digitalWrite(yellowLED, LOW); noTone(buzzer);

// Condition 1: Temperature > 70 F if (temperatureF > 70) { digitalWrite(greenLED, HIGH);

}

// Condition 2: 30 F < Temperature < 70 F

else if (temperatureF > 30 && temperatureF < 70) { digitalWrite(yellowLED, HIGH);

}

// Condition 3: Temperature < 30 F else if (temperatureF < 30) { tone(buzzer, 1000);

}

delay(1000);

}

## Output:

Condition 1: temp is > 70-degree F green light is in ON

Condition 2: temp is<70-degree F and temp is> 30-degree F yellow light is in ON

Condition 3: temp is < 30 degree F turn on the buzzer and the lights will remain OFF

## Learning Outcomes :

- Interface a TMP36 temperature sensor with an Arduino Uno.

- Read the analog output of the TMP36 and calculate the temperature in Fahrenheit.

- Display the calculated temperature on the Serial Monitor and I2C LCD.

- Control green and yellow LEDs and a buzzer based on predefined temperature conditions.

- Implement a basic temperature monitoring and alert system using Arduino Uno.